# Trykktank (Pressure Tank) — SolidWorks Cookbook

## Dimensions

| Parameter | Value |
| --- | --- |
| Inner diameter | 200 mm |
| Wall thickness | 5 mm |
| Cylinder height | 600 mm |
| Dome | Hemisphere on top |
| Hole at dome apex | Ø 20 mm |
| Bottom plate | 5 mm thick (flat) |
| Material | Aluminum 1060 Alloy |
| Pressure | 10 bar (= 1 MPa) |

## A. Build the Part

### A1 — New Part + Units

- File → New → Part → OK

- Bottom-right corner → set units to MMGS

### A2 — Sketch on the Right Plane

- In the FeatureManager tree, click Right Plane

- Right-click → Sketch

### A3 — Draw the Centreline (revolve axis)

- Tools → Sketch Entities → Centerline

- Click the origin → drag straight up → click to place

- This vertical line is the revolve axis

### A4 — Draw the Outer Profile

Draw 3 lines + 1 arc to the right of the axis:

AXIS (centreline)
           |
           |
    (0,710)+ _ _
           |     ' .
           |         '.       quarter-circle arc
           |           \      centre = (0, 605)
           |            \     radius = 105 mm
           |             |
           |             | (105, 605)
           |             |
           |             |    vertical line
           |             |    605 mm tall
           |             |
           |             |
           |             |
    (0, 0) +-------------+ (105, 0)
         origin

           <-- 105 mm -->

- Line (L): origin (0, 0) → right to (105, 0)

- Line: (105, 0) → up to (105, 605)

- Tangent Arc: (105, 605) → curve up-and-left to (0, 710)

  - Tools → Sketch Entities → Tangent Arc

  - Click at the top of the vertical line, drag upward and to the left

  - The arc must end on the axis (X = 0)

The profile closes automatically along the centreline.

### A5 — Add Dimensions

- Smart Dimension (D):

  - Bottom line → 105

  - Vertical line → 605

  - Arc radius → 105

- All lines should turn black (fully defined)

### A6 — Revolve

- Features → Revolved Boss/Base

- Axis = the centreline (auto-detected)

- Angle = 360° → click ✓

Result: a solid cylinder with a hemisphere dome. Not hollow yet.

### A7 — Shell (hollow it out)

- Features → Shell

- Faces to remove: click the flat bottom face (disc at the bottom)

- Wall Thickness: 5

- Click ✓

Result: the body is now a 5 mm hollow shell — walls, dome, and bottom plate.

### A8 — Cut the 20 mm Hole

- Click Top Plane → start a new Sketch

- Draw a Circle at the origin, radius 10 mm

- Exit the sketch

- Features → Extruded Cut

- Direction: Up To Next (cuts only the dome, not the bottom)

  - Arrow must point downward. If it points up → Reverse Direction

  - Do NOT use “Through All” — it cuts both top and bottom!

- Click ✓

### A9 — Assign Material

- Right-click Material → Edit Material

- Aluminum Alloys → 1060 Alloy → Apply → Close

### A10 — Save

File → Save As → PressureTank.SLDPRT

## B. Run FEA Simulation (10 bar)

### B1 — New Study

- Simulation tab → New Study

- Name: Pressure_10bar, Type: Static → ✓

### B2 — Fix the Bottom

- Right-click Fixtures → Fixed Geometry…

- Click the flat bottom face → ✓

### B3 — Apply Pressure on Interior Faces

First, make the inside visible:

- View → Display → Section View → pick Right Plane → ✓

Then apply pressure:

- Right-click External Loads → Pressure…

- Hold Ctrl and click all inner faces:

  - Inner cylinder wall

  - Inner dome

  - Inner bottom plate

- Value: 1000000 Pa (= 10 bar = 1 MPa)

- Arrows should point outward. If inward → Reverse direction

- Click ✓

- Turn off Section View: View → Display → Section View (toggle)

### B4 — Mesh + Run

- Right-click Mesh → Create Mesh → ✓

- Right-click study name → Run

### B5 — Read Results

| Result | How to read |
| --- | --- |
| Max stress | Double-click Stress plot → read max from legend |
| Max displacement | Double-click Displacement plot → read max |
| Min Factor of Safety | Right-click Results → Define Factor of Safety Plot → ✓ → read min from legend |

Question: Is min FoS ≥ 2? If not, the wall is too thin.

## C. Parametric Study (thickness vs FoS vs cost)

### C1 — Fill this Table

Repeat the simulation for each wall thickness:

| Thickness (mm) | Max Stress (MPa) | Min FoS | Mass (kg) | Cost ($) at $2.70/kg |
| --- | --- | --- | --- | --- |
| 3 | 33.3 | 0.83 | 3.89 | 10.51 |
| 4 | 25.0 | 1.10 | 5.23 | 14.11 |
| 5 | 20.0 | 1.38 | 6.58 | 17.75 |
| 6 | 16.7 | 1.66 | 7.94 | 21.44 |
| 7 | 14.3 | 1.93 | 9.33 | 25.18 |
| 8 | 12.5 | 2.21 | 10.73 | 28.97 |
| 9 | 11.1 | 2.48 | 12.15 | 32.80 |
| 10 | 10.0 | 2.76 | 13.59 | 36.68 |

Values computed using thin-wall pressure vessel theory: σ = p×r/t (cylinder hoop stress). Yield strength of 1060 Alloy ≈ 27.6 MPa. FEA results may differ slightly due to stress concentrations at the dome-cylinder junction and the hole.

Key rows bolded: t=5 mm (original design) and t=8 mm (first thickness where FoS ≥ 2.0).

### C2 — How to Change Thickness

For each row:

- Go to the Model tab (exit Simulation)

- In the FeatureManager, expand Shell1

- Double-click it → change Wall Thickness to the new value → ✓

- Ctrl+Q to rebuild

- Check mass: Tools → Evaluate → Mass Properties → read Mass

- Cost = mass × $2.70

- Go to Simulation tab → Run

- Read the new FoS from the Factor of Safety plot

- Record in the table

### C3 — Make the Graph

- Open Excel

- Enter: Thickness, FoS, Cost

- Insert a combo chart:

  - X-axis: Wall Thickness

  - Left Y-axis: Factor of Safety (line)

  - Right Y-axis: Cost (line or bars)

- Add a horizontal line at FoS = 2.0

### C4 — Cost Comparison

|  | Original design | Safe design |
| --- | --- | --- |
| Wall thickness | 5 mm | 8 mm |
| Min FoS | 1.38 | 2.21 |
| Mass | 6.58 kg | 10.73 kg |
| Material cost | $17.75 | $28.97 |

% cost increase = (28.97 − 17.75) / 17.75 × 100 = 63.2%

The original tank (t = 5 mm) does not meet the FoS ≥ 2.0 requirement (FoS = 1.38). The minimum wall thickness for FoS ≥ 2.0 is 8 mm (FoS = 2.21), which increases material cost by 63% — from $17.75 to $28.97.

## Troubleshooting

| Problem | Fix |
| --- | --- |
| Body is solid, not hollow | You forgot Shell (step A7). Features → Shell → select bottom face → 5 mm |
| Hole goes through both sides | You used “Through All”. Undo → redo with Up To Next |
| Cut arrow points wrong way | Click Reverse Direction |
| Can’t select inner faces | Use Section View (View → Display → Section View) |
| Pressure arrows point inward | Click Reverse direction in the Pressure dialog |
| Sketch lines are blue | Add more dimensions (D) until all lines are black |
| Arc won’t connect to axis | Add a Coincident relation between arc endpoint and centreline |