# Trykktank — Simplest Build Method

Revolve a solid shape, then use Shell to hollow it out, then cut the hole.

## Step 1 — New Part

- File → New → Part → OK

- Set units to MMGS (bottom-right corner)

## Step 2 — Sketch on the Right Plane

- Click Right Plane in the FeatureManager tree

- Right-click → Sketch

## Step 3 — Draw a Vertical Centreline

- Tools → Sketch Entities → Centerline

- Click the origin → drag straight up → click to place

- This is the revolve axis

## Step 4 — Draw the Outer Half-Profile

Draw only 3 lines + 1 arc — the simple outer shape:

AXIS (centreline)
           |
           |
    (0,710)+ _ _
           |     ' .
           |         '.        quarter-circle arc
           |           \       centre = (0, 605)
           |            \      radius = 105 mm
           |             |
           |             |  (105, 605)  <-- arc starts here
           |             |
           |             |     vertical line
           |             |     605 mm tall
           |             |
           |             |
           |             |
    (0, 0) +-------------+  (105, 0)
         origin        bottom-right

    <--- 105 mm --->

    When you REVOLVE this 360 deg around the axis:
    - bottom line   -->  flat disc (bottom plate)
    - vertical line -->  cylinder (radius 105)
    - arc           -->  hemisphere dome

Important — the axis is the LEFT edge. The profile hangs to the RIGHT of the axis. The arc curves from the top of the wall back to the axis — that quarter-circle becomes a hemisphere when spun 360°.

- Line (L): From (0, 0) → right to (105, 0) — bottom edge

- Line: From (105, 0) → up to (105, 605) — outer wall

- Tangent Arc: From (105, 605) curving up-and-left to (0, 710) — dome

  - Use Tools → Sketch Entities → Tangent Arc

  - Start clicking at (105, 605) then drag up and to the left

  - The arc must end on the axis (X = 0) at Y = 710

  - Arc centre is at (0, 605), radius = 105

- The profile closes automatically along the centreline (the axis side)

Dimensions (Smart Dimension, D):

- Bottom line: 105 mm

- Vertical line: 605 mm

- Arc radius: 105 mm

All lines should turn black (fully defined).

## Step 5 — Revolve (solid body)

- Features → Revolved Boss/Base

- Axis = the centreline (usually auto-detected)

- Angle = 360°

- Click ✓

You now have a solid cylinder + dome. It is not hollow yet — that’s fine.

## Step 6 — Shell (make it hollow)

This is the key step:

- Features → Shell (or Insert → Features → Shell)

- It asks: “Select faces to remove”

- Click the flat bottom face (the circular disc at the very bottom)

- Set Wall Thickness = 5 mm

- Click ✓

Done — the tank is now hollow with 5 mm walls everywhere (cylinder, dome, and bottom plate).

## Step 7 — Cut the 20 mm Hole at the Top

- Click Top Plane in the FeatureManager

- Start a new Sketch on it

- Draw a Circle centred at the origin, radius = 10 mm (diameter 20 mm)

- Exit sketch

- Features → Extruded Cut

- Set direction to Up To Next

  - NOT “Through All” — that cuts both top and bottom!

  - Arrow must point downward into the dome. If it points up, click Reverse Direction

- Check the preview — only the dome apex should be cut

- Click ✓

## Step 8 — Material

- Right-click Material → Edit Material

- Expand Aluminum Alloys → select 1060 Alloy

- Apply → Close

## Step 9 — Save

File → Save As → PressureTank.SLDPRT

## Common Mistakes

| Problem | Cause | Fix |
| --- | --- | --- |
| Tank is solid, not hollow | Forgot the Shell step | Features → Shell, select bottom face, 5 mm |
| Hole goes through both sides | Used “Through All” in Extruded Cut | Undo (Ctrl+Z), redo with Up To Next |
| Cut arrow points wrong way | Direction is reversed | Click Reverse Direction in the cut dialog |
| Arc won’t connect to axis | Endpoint not on axis | Drag endpoint to X=0 on the centreline, or add a Coincident relation |

## Summary

| What | How |
| --- | --- |
| Outer shape | 3 lines + 1 arc → Revolve 360° |
| Hollow it out | Shell → select bottom face → 5 mm |
| Top hole | Circle Ø20 mm → Extruded Cut → Up To Next |
| Material | 1060 Alloy (Aluminum) |